Instructions for setting audio receivers to locate calling partridge

Description of the receivers (sensors)
The receivers are ‘passive acoustic monitors’ (PAM) that can be deployed in the field to record wildlife sounds onto an SD card. Each receiver has two microphones, extending from its left and right sides, so that the direction of the sound to the sensor can be calculated. The sensors all contain a GPS which ensures that their internal clocks are perfectly synchronised with each other. The GPS also records the geographic position (easting and northing) of the receivers.

How to programme the receivers

All receivers  contain a GPS which ensures that their internal clocks are syncronised. However, it is essential that all receivers are programmed to to record during identical time periods.

Partridges call most frequently around dawn and dusk, we suggest using the settings on all sensors to record for 1 hour starting at dawn and one hour finishing at dusk each day and leaving the sensors in place for 7-10 days. The twice-daily 1 hour recording periods can be set via the 24 hour control panel on the front of the receiver.

How to deploy the sensors in the field

Sensors should be laid out in a pattern so that at least two receivers and preferably three will pick up calling partridges in the vicinity.  The distance between sensors will be determined by the distance over which calls can be reliably heard.  In this section we assume that this distance is 200m but this can be confirmed by early trials on the rearing field and be adjusted if required.

For this trial, it would be best to select an area with a particularly high local density of partridge territories. This is likely to involve linear features such as hedges or cover crops.

Assuming they are to be set along a linear feature such as a hedge.  Set them out approximately100m apart into the field on alternate sides of the hedge at intervals of 100m (see diagram below. If a map of the site is available we can adapt this pattern as needed

How to set up each sensor

Each sensor should be attached to the top of a short wooden stake so that it is held approximately 1m above the ground and above the top of the crop.  The stakes should be secure so that they do not move with the wind. Use an indelible waterproof pen to clearly mark each sensor with the numbers 1 to 6.

When each sensor is put in place, record the GPS location, there is a built in GPS display in each sensor. Position each sensor so that the front of the receiver is facing due north, i.e. the direction of the arrow (diagram below) is point north on a hand-held magnetic compass or mobile-phone compass. Note: In exceptional circumstances it might not be possible to position the receiver so that the front is pointing north. Hence, always, record the angle from north in a clockwise direction (N = 0, E = 90, S = 180, W = 270). However, wherever practicable, have the front of the receiver pointing north.

Data Recording Form

Use this form to record the data. If collecting over multiple weeks, complete a new form for each week.

Date receivers set in field …………………

Date receivers collected from field ……………….

Location…………………

Start time of dawn recording (hr:min):………… End time of dawn recording:……………..

Start time of dusk recording (hr:min):…………..End time of dusk recording:……………..

| Sensor number | GPS Location | Angle set |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |
| 6 |  |  |

SD Card Handling
At the end of the trial, carefully remove each SD Card from its sensor and put into a small plastic bag that also contains a piece of paper with the Sensor Number (1 to 6), using a separate bag and label for each SD Card. Small freezer bags with an integral zip closure are convenient.